ARVR Lab Assignment No. 1

Submitted by:

| Sr. No. | PRN | Student Name |
| --- | --- | --- |
| 1 | 1032233145 | Ibrahim Ansari |

Title: Comparison of Various Augmented Reality and Virtual Reality hardware devices

Aim: To study and compare various Augmented Reality and Virtual Reality devices on various attributes

Comparison of Augmented Reality and Virtual Reality Devices:

| Specification / Model | Meta Quest 3 | Apple Vision Pro | Microsoft HoloLens 2 |
| --- | --- | --- | --- |
| Device manufacturer with Launch Date | Meta (October 2023) | Apple (February 2024) | Microsoft (November 2019) |
| Device form factor | Standalone VR/MR Headset | Standalone Spatial Computer (Tethered Battery) | Standalone AR Headset (Visor style) |
| Processing capability | Qualcomm Snapdragon XR2 Gen 2 | Apple M2 (Main) + Apple R1 (Sensor processing) | Qualcomm Snapdragon 850 + Custom HPU 2.0 |
| Number of Sensors on the Device | Approx. 7 sensors (external) + IMUs | Approx. 12 cameras + 5 sensors | Approx. 9 sensors + Microphone array |
| Names of Sensors on the Device | 2 RGB Cameras, 4 IR Tracking Cameras, 1 Depth Sensor, IMU | 2 Main Cameras, 6 Tracking Cameras, 4 Eye Cameras, TrueDepth, LiDAR, IMUs, Flicker, Ambient Light | 4 Visible Light Cameras (Head tracking), 2 IR Cameras (Eye tracking), 1-MP Time-of-Flight (Depth), IMU, 8MP RGB Camera |
| Platform of the device | Meta Horizon OS (Android-based) | visionOS | Windows Holographic |
| GPS Enabled | No (Relies on SLAM/Wi-Fi) | No (Relies on SLAM/Wi-Fi) | No (Relies on SLAM/Wi-Fi) |
| Degree of Freedom | 6 DoF | 6 DoF | 6 DoF |
| Display Type | LCD (2 LCDs) | Micro-OLED | See-through Holographic Lenses (Waveguides) |
| Resolution | 2064 x 2208 pixels per eye | ~3660 x 3142 pixels per eye (23M pixels total) | 2k 3:2 light engines (1440 x 936 per eye equivalent) |
| Field of View (in degrees) | ~110° Horizontal, 96° Vertical | ~100° Horizontal | ~52° Diagonal |
| Pass through Camera | High-fidelity Color Video Passthrough | Ultra-low latency Color Video Passthrough | N/A (Optical See-Through - Real world is seen through glass) |
| Primary / Major application domains | Gaming, Social VR, Fitness, Light MR | Productivity, Media Consumption, Spatial Computing, Collaboration | Enterprise, Manufacturing, Training, Medical, Engineering |
| Dedicated SDK available for application development? | Yes (Meta XR SDK, Unity, Unreal) | Yes (visionOS SDK, ARKit, SwiftUI) | Yes (MRTK - Mixed Reality Toolkit, Unity, Unreal) |
| Advantages | Affordable price, excellent standalone performance, vast game library, wireless freedom. | Incredible display fidelity, seamless ecosystem integration, intuitive eye/hand tracking without controllers. | Hands-free optical AR (natural view of world), comfortable for long wear (balanced weight), flip-up visor. |
| Disadvantages | Battery life (approx. 2 hours), LCD black levels are not perfect, heavier front balance. | Very expensive, external battery pack, weight can cause fatigue, limited VR gaming library. | Narrow field of view (window effect), older processor, lower resolution compared to modern VR, expensive. |
| Headset Weight | 515g | ~600-650g | 566g |
| Connection Type | USB-C (Charging/Link), Wi-Fi 6E | USB-C (On Battery Pack), Wi-Fi 6 | USB-C, Wi-Fi 5, Bluetooth 5.0 |

Comments / Discussion :- After the comparative study of the headsets indicated above, it is observed that Meta Quest 3 is suitable for low-cost VR experiences, whereas the Apple Vision Pro is suitable for VR experiences requiring more rendering capabilities. The Microsoft HoloLens is more focused towards AR experiences.

Conclusion: Thus, analysed different VR/AR headsets and their suitable use-cases and capabilities.

FAQs:

- What is the field of view? Preferred Field of view should be less or more?

- What is the degree of freedom? Why does it matter?

- What are various types of displays available for Augmented Reality and Virtual Reality?

- What is the device form factor? How does it affect the usability of the device?